# Jack Wolowacz - 20268056

# Drawing Robot Software – Final Project Submission

# Software Description

The program created should take in an input of a text file containing a message to be written by the robot. The program should then read the input and generate gcode instructions that will be sent to a buffer that an Arduino will use to control the robot.

The program should begin by opening the input and font files and loading the font file into memory, saving each character in an array as a set of pen strokes. Whenever reading a file, the program should check that the file exists. If the file does not exist, then an error should be thrown.

Figure 1 - Example input text file

The program should then initialise the arm (send commands to buffer for setting pen speed, turn on spindle and put the pen up) before asking the user for the desired font size, scaling between text height of 4-10mm. The program then should read the first word of the input file and store it in an array. The program should then look up the first letter in the first word in SingeStrokeFont.txt and find the corresponding series of instructions. The coordinates found in the instructions should be scaled to the size determined by the user. The program should then proceed to the next letter in the word, appending the new instructions into an array of pen stroke commands. Each character in the word should be written from the reference point defined in the previous character.

Figure 2 - Example output using gcode simulator. font size is 5mm and input text file is shown in Figure 1

The program should ensure that if a word is going to spill over the defined page width of 100mm, that the word will be written on the next line by moving the local word origin. If a word is longer than the page width on its own, the program should display an error. If the program detects a LF (ASCII 10) or CR (ASCII 13) character in the input, the program should place the word after on a new line, regardless of space left on the line. The instructions should be converted into a series of gcode instructions which should then be sent one by one into a buffer. Once completed, the program should proceed with the next word in the file. Once a line is completed, the program should wrap the next word onto the next line. A gap of 5mm should be included between each line regardless of font size. The program should repeat this until the end of the file is reached, at which point the program will terminate.

# Project Files

## Library Files

/lib/logger

/lib/logger contains log.h, which declares all the public logging related functions, and log.c, which defines these functions. This includes writing the gcode commands to a log file so larger input text files can be used, as the terminal only allows so many lines to be shown.

/lib/rs232

/lib/rs232 contains the rs232 library files, rs232.h and rs232.c. These files, developed by Teunis van Beelen, declare and define functions needed to open, send to and receive data from, and close a serial port connection.

## Source Files

/src/main.c

/src/main.c contains the main function. This file defines the main flow of the program, featuring the main loop and initialisation and cleanup tasks.

/src/InputHandling

/src/InputHandling contains InputHandling.h and InputHandling.c, which declares and defines functions handling both user and file inputs, including inputting and filtering font sizes, reading information from input and font files and saving the font data into a structure.

/src/RobotControl

/src/RobotControl contains RobotControl.h and RobotControl.c, which declares and defines functions handling robot initialisation, shutdown, command handling and gcode generation.

/src/SerialOutput

/src/SerialOutput contains serial.h and serial.c, which declares and defines functions handling the serial communication sent to the RS232 port using the RS232 library.

## Input Files

All files in the input files folder are copied to the directory the Drawing Robot Driver will be compiled to as a post-build step in CMake.

/src/InputFiles/SingleStrokeFont.txt

Font file. Program will open this file and use it to define the gcode for each character.

/src/InputFiles/test.txt

Input file. Program will read the text in this file and convert it to gcode commands to send to the Arduino.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| Vertex | Struct: float x float y | Structure to represent a position in 2d space. One float is used to represent each coordinate in a 2D plane |
| PenStroke | Struct: Struct Vertex NewPosition bool PenUpStatus | Structure to represent the stroke of the pen. A vertex struct is used to represent the position at the end of the stroke, and an integer is used to represent whether the pen is up or down. |
| PenStrokeSeries | Struct: int StrokeCount struct Vertex LocalOrigin struct PenStroke* StrokeData | Structure to contain a series of sequential pen strokes. The number of strokes stored is saved in the integer StrokeCount. A dynamically allocated pointer is used to store an arbitrary number of strokes, and a vertex is used to represent the local origin. |
| CharData | Struct: Char characterEncoded Struct PenStrokeSeries CharacterStrokes | All data from font file needed to draw a character. characterEncoded is the character that the PenStrokeSeries contains all the pen strokes required to draw. |
| FontData | CharData[128] | An array of CharData structs. Each CharData contains all the information needed to draw a character. The correct CharData struct can be found in this array by using the ASCII code of a character as the index in the array. |
| GCode GeneratorInput | Struct: char* inputWord float fontSize struct Vertex origin; struct CharData* fontData | A struct that gives all data from font file needed to generate the gcode for a single word. |

# Functions

## Input Handling

float AskUserForFontSize();

Asks user for a font size between 4 and 10mm. Repeats if user inputs a number outside this range or an invalid value (anything that contains a letter).

Return value (float)	Font size value between 4 and 10.

FILE* LoadFileFromPath(char* filePath);

Loads a file to read from a path specified.

filePath (char*) 		The path to a file to be opened.

Return value (FILE*)		file pointer if successful, null pointer if failed. Fatal log message displays error.

void LoadFontDataFromFile(FILE* fontFile, struct CharData FontData[]);

Loads font data from file into an array of structs. Pass ascii code as index to receive PenStroke data.

fontFile (FILE*)		A pointer to the font file.

FontData (FontData*)	This pointer will contain the data read from the font file.

Return value		void.

void ReadWordFromInputFile(FILE* inputFile, char* inputWord);

Reads single word from file.

inputFile (FILE*)		A pointer to the input file.

inputWord (char*)	Will be modified to contain the word read from the file.

Return value		void.

int GetNextCharacterCode(FILE* inputFile);

Reads single character from file.

inputFile (FILE*)		A pointer to the input file.

Return value (int)		ASCII character code.

## Robot Control

int InitialiseRobot();

Initialises robot. Opens RS232 port and sends necessary commands to wake up and initialise the robot.

Return value (int)		0 if successful, -1 if failed.

void ShutdownRobot();

Shuts down robot. Moves pen up and back to global origin.

Return value		void.

int CheckWordFitsOnPage(char* inputWord, float fontSize, struct Vertex Origin);

Checks if word can fit on page based on current position of end of previous word.

inputWord (char*) 	The word that needs to fit on the page.

fontSize (float)		The size of the font the user selected.

Origin (struct Vertex)	The position of the end of the previous word.

Return value (int)		0 if word can fit, -1 if word cannot fit.

int GenerateGCodeForWord(struct GCodeGeneratorInput* input);

Generates gcode for a word and sends commands to buffer.

input (struct GCodeGeneratorInput*) Pointer to input struct that contains all input information.

Return value (int)		 0 if successful, -1 if failed.

int GenerateGCodeForLetter(struct PenStrokeSeries LetterData, float fontSize);

Generates gcode for a single letter and sends commands to buffer.

LetterData (struct PenStrokeSeries) All the data needed to draw a single character.

fontSize (float) 		Font size inputted by user.

Return value		0 if successful, -1 if failed.

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| AskUserFor FontSize | Integer between 4 and 10 | User Input: 4 | 4.0 |
|  | Float between 4 and 10 | User Input: 5.6 | 5.6 |
|  | Integer not between 4 and 10 | User Input: 99 | Error: Invalid font size. Re-enter value. |
|  | Float not between 4 and 10 | User Input: 2.5 | Error: Invalid font size. Re-enter value. |
|  | String | User Input: hello | Error: Invalid font size. Re-enter value. |
| LoadFile FromPath | Font file | filePath: SingleStrokeFont.txt | File pointer to font file. |
|  | Input file | filePath: test.txt | File pointer to input file. |
|  | Non-existent file | filePath: random.txt | Fatal: Could not open file random.txt Null pointer. |
| LoadFontData FromFile | Font file | fontFile: pointer to SingleStrokeFont.txt | CharData FontData[128] filled. |
|  | Input file | fontFile: pointer to test.txt | Fatal: File format is invalid! FontData set to null. |
|  | Non-existent file | fontFile: null pointer | Fatal: File could not be opened! FontData set to null. |
| ReadWordFrom InputFile | Regular string | inputFile: pointer to test.txt test.txt: “The quick brown fox jumps over the lazy dog.” | Void. inputWord set to “The”. |
|  | Long word | inputFile: pointer to test.txt test.txt: “Theeeeeeeeeeeeeeeeeeeeeeeee” | Error: Word is too long! inputWord set to null. |
|  | Null pointer | inputFile: null pointer | Fatal: File could not be read! inputWord set to null. |
| GetNext CharacterCode | Regular string | inputFile: pointer to test.txt test.txt: “The quick brown fox jumps over the lazy dog.” | 84 |
|  | Space | inputFile: pointer to test.txt test.txt: “ Hello!“ | 32 |
|  | EOL | inputFile: pointer to test.txt test.txt: “ Hello!” | 10 |
| InitialiseRobot | Standard | - | 0 |
| Shutdown Robot | Standard | - | Void |
| CheckWord FitsOnPage | Word fits on page | inputWord: “Hello!” fontSize: 6 Origin: [x: 0, y: 0] | 0 |
|  | Word does not fit on page | inputWord: “Helloooooo” fontSize: 10 Orign: [x: 0, y: 0] | -1 |
|  | Word does not fit in remaining page | inputWord: “Hello!” fontSize: 4 Origin: [x: 90, y: 0] | -1 |
| GenerateGCode ForWord | Normal word input | GCodeGeneratorInput (“Hello”, 4, (0,0), fontData) | HelloGcode.txt |
|  | Undefined character | GCodeGeneratorInput (“¬!”£$%^&*()”, 4, (0,0), fontData) (¬ and £ are undefined” | Error: Character not supported! inputWord set to null. |
| GenerateGCode ForLetter | Normal letter input | FontData[84] - > Value for font data for T | TGcode.txt – Gcode output |
|  | Space input | FontData[30] - > value for font data for space | SpaceGCode.txt – Gcode output |
| main | Standard input | test.txt: “The quick brown fox jumps over the lazy dog” | Result1.txt – Gcode output |
|  | New line input | test.txt “The quick brown fox jumps over the lazy dog” | Result2.txt – Gcode output |
|  | Long input | test.txt “The speedy brief nippy quick brown fox jumps over the very extremely entirely wholeheartedly lazy dog.” | Result3.txt – Gcode output |
|  | Non-existent file | Thisdoesntexist.txt | Fatal: Could not open file Thisdoesntexist.txt |
|  | Undefined characters | test.txt: “¬!”£$%^&*()” | Error: Character not supported! |

# AI Statement

No LLMs or other AI were used in the production of this report nor any work presented.